| CODE | DESCRIPTION | QTY | UNIT |
| --- | --- | --- | --- |
| BAZIAN FRUIT PROCESSING PLANT - Grape Juice treatment line (5 t/h) |  |  |  |
| 01-08-00057 | Washing/extraction unit: complete line with tilting machine for cases; washing device; elevator; crusher/destemmer machine; screw conveyor for wastes; pumps; mixing tank; pump; press (membrane type); fittings; electric board. All parts in contact with product made in staniless steel. | 1 | Item |
| 01-08-00058 | Vibroscreener: to screen, separate, filter, by adjustable vibration, fruit pulp, juice, etc. | 1 | Item |
| 01-08-00059 | 300 L tank: made in stainless steel AISI 304. Sloped bottom, level regulator for the automatic start-ut and stop of the servoce pump. | 1 | Item |
| 01-08-00060 | Sanitary volumetric pump: made in stainless stee, suitable for food products.Positive pump, single screw type with stainless steel screw rotor and food grade rubber stator. | 1 | Item |
| 01-08-00061 | Pasteurizer: Tubular heat exchanger to pasteurize fruit juices. Cooling section, hot water preparation group, fittings, electric board. All parts in contact with product made in staniless steel. | 1 | Item |
| 01-08-00062 | 5000 L mixing tank: for juice clarification by enzyme, bentonite, gelatine. Made in stainless steel, with conical bottom, loading and discharging taps, washing ball, disharging throttle valve, stirrer, supporting legs. | 4 | Item |
| 01-08-00063 | 200 L mixing tank: for adding/dosing additives (enzymes, bentonite, gelatine). Equipped with loading and discharging taps, washing ball, disharging throttle valve, stirrer, supporting legs. | 2 | Item |
| 01-08-00064 | Dosing pump: for additive dosage. Made in stainless steel. | 1 | Item |
| 01-08-00065 | Sanitary volumetric pump: made in stainless stee, suitable for food products.Positive pump, single screw type with stainless steel screw rotor and food grade rubber stator. | 1 | Item |
| 01-08-00066 | Filter:using fossile flour as filtering means. | 1 | Item |
| 01-08-00067 | 5000 L tank: made in stainless steel, with conical bottom, loading and discharging taps, washing ball, disharging throttle valve, supporting legs. | 1 | Item |
| 01-08-00068 | Sanitary volumetric pump: made in stainless stee, suitable for food products.Positive pump, single screw type with stainless steel screw rotor and food grade rubber stator. | 1 | Item |
| 01-08-00069 | Cooler: tubular echanger to refrigerate the juice up to 2 to 0 °C. Refrigerating group and fittings. All parts in contact with product made in stainless steel. | 1 | Item |
| 01-08-00070 | Electric control board: made in stainless steel, with door in order to allow a complete internal inspection. The unit has to include main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. | 1 | Item |
| 01-08-00071 | Fittings: Accessories, connections, taps, valves, piping, joints, reducers, bends, tees, manual sanitary valves, pipe holders and support etc. made in stainless steel to connect utilities to the process equipment. Working platforms are also included. | 1 | Set |
| BAZIAN FRUIT PROCESSING PLANT - Grape Juice Cold Store (0°C) |  |  |  |
| 01-08-00072 | 20,000 L tank: made in glass-resin, or equivalent material, for detartarization activity (sedimentation at low temperature for 7 days). Equipped with loading and discharging taps, washing ball, disharging throttle valve, inspection man-hole, supporting legs. All required material for tanks inter-connections. | 30 | Item |
| 01-08-00073 | 550 m2 Prefabricate Structure: cold store type, to maintain the temperature of the juice into the 30 tanks at (0 to +2 °C). Refrigerating group and all required instrumentation, electric connections, fittings and accessories. | 1 | Set |
| 01-08-00074 | Centrifugal pump: to move and transfer fruit juice. All part in contact with product made in stainless steel. | 2 | Item |
| BAZIAN FRUIT PROCESSING PLANT - Detartarized Grape Juice Preparation and Concentration line |  |  |  |
| 01-08-00075 | Filter: rotating drum type using fossile flour as filtering means. | 1 | Item |
| 01-08-00076 | 5000 l tank: made in stainless steel, with conical bottom, loading and discharging taps, washing ball, disharging throttle valve, supporting legs. | 1 | Item |
| 01-08-00077 | Filter:plate filter type utilising mill board (carton) cartridge as filtering means. | 1 | Item |
| 01-08-00078 | 1000 L tank: made in stainless steel, with conical bottom, loading and discharging taps, washing ball, disharging throttle valve, supporting legs. | 1 | Item |
| 01-08-00079 | Sanitary volumetric pump: made in stainless stee, suitable for food products.Positive pump, single screw type with stainless steel screw rotor and food grade rubber stator. | 1 | Item |
| 01-08-00080 | Grape juice plate evaporator: double effect, falling-film type to concentrate grape juice up to 70°B. Equipped with: thermo-compressor; centrifugal pump for CIP; plate heat exchanger for pre-heating and pasteurization purpose; no. 2 plate heat echangers falling-film type; no.2 cylindrical evaporation chambers; horizontal surface condenser with tube nest; electronic refractometer with automatic adjusting; pumps for juice recycling (evaporation cycle), concentrate extraction, condensates return and vacuum system. Part in contact with the product made in AISI 304; evaporate surfaces in AISI 316 | 1 | Item |
| 01-08-00081 | Electric control board: made in stainless steel, with door in order to allow a complete internal inspection. The unit has to include main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. | 2 | Item |
| 01-08-00082 | Fittings: Accessories, connections, taps, valves, piping, joints, reducers, bends, tees, manual sanitary valves, pipe holders and support etc. made in stainless steel to connect utilities to the process equipment. Working platforms are also included. | 1 | Set |
| BAZIAN FRUIT PROCESSING PLANT - Spare parts |  |  |  |
| 01-08-00083 | Spare parts: According to the experience of the equipment manufacturer, required spare parts will be provided. Reference is made to processing equipment, laboratory, utilities system and miscellaneous equipment. | 1 | Lot |

| 01-08-00084 | 500 L S.S. mixing tank: for dosing bentonite, with accessories | 2 | Unit |
| --- | --- | --- | --- |
| 01-08-00085 | Dosing pump: for additive dosage | 1 | Unit |