ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link Between  BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name :  MAMOUN

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-01100 07-03-01101 07-03-01102 07-03-01103 | SDH Microwave Radio Equipment, STM-1, 13GHZ (2+1), (3+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 3 3 3 1 |  |
| 07-03-01104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 3 |  |
| 07-03-01105 07-03-01106 07-03-01107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 3 1 1 |  |
| 07-03-01108 07-03-01109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(10+2) config. 48V, 1000Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-01110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-01111 07-03-01112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 0 0 |  |
| 07-03-01113 07-03-01114 07-03-01115 07-03-01116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM- 4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoding-Decoding Equipment 34Mb/s ip/op, Access & Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subracks with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-01117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-01118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-01119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-01120 07-03-01121 07-03-01222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 4 4 2 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension To Port  UM QASR

Site - Name :  SINEK

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-02100 07-03-02101 07-03-02102 07-03-02103 | SDH Microwave Radio Equipment, STM-1, 13GHZ (2+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-02104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-02105 07-03-02106 07-03-02107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-02108 07-03-02109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module (10+2) config. 48V, 1000Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-02110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-02111 07-03-02112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 0 0 |  |
| 07-03-02113 07-03-02114 07-03-02115 07-03-02116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM- 4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoder-Decoder Equipment 34Mb/s ip/op. Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-02117 | -Network Management Equipment consisting of Central Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-02118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-02119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-02120 07-03-02121 07-03-02222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 2 4 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital Microwave Link  Between  BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name :  BAYAA  Microwave  Station

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-03100 07-03-03101 07-03-03102 07-03-03103 | SDH Microwave Radio Equipment, STM-1, 11,8GHZ (3+1) Frequency, Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-03104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 3 |  |
| 07-03-03105 07-03-03106 07-03-03107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 3 1 1 |  |
| 07-03-03108 07-03-03109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(5+1)configuration, 48V, 250Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-03110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-03111 07-03-03112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-03113 07-03-03114 07-03-03115 07-03-03116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-03117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-03118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-03119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-03120 07-03-03121 07-03-03222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 2 4 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAF & BASRAH  With

Extension  To Port  UM QASR

Site - Name : MUSSAIEB

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-04100 07-03-04101 07-03-04102 07-03-04103 | SDH Microwave Radio Equipment, STM-1, 11,6,8 GHZ(1+1) ,(3+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 4 4 4 1 |  |
| 07-03-04104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 7 |  |
| 07-03-04105 07-03-04106 07-03-04107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 7 1 1 |  |
| 07-03-04108 07-03-04109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(4+1)configuration, 48V, 250Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-04110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-04111 07-03-04112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-04113 07-03-04114 07-03-04115 07-03-04116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-04117 | -Network Management Equipment consisting of Local Network Management System platform (Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-04118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-04119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-04120 07-03-04121 07-03-04222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector  : Digital Microwave Link  Between BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name  : HILLA

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-05100 07-03-05101 07-03-05102 07-03-05103 | SDH Microwave Radio Equipment, STM-1, 8GHZ (3+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-05104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 4 |  |
| 07-03-05105 07-03-05106 07-03-05107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 4 1 1 |  |
| 07-03-05108 07-03-05109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(3+1)configuration, 48V, 250Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-05110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-05111 07-03-05112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-05113 07-03-05114 07-03-05115 07-03-05116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-05117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-05118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-05119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-05120 07-03-05121 07-03-05222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital Microwave Link  Between  BAGHDAD & BASRAH With

Extension To Port  UM QASR

Site - Name  : KUFFA

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-06100 07-03-06101 07-03-06102 07-03-06103 | SDH Microwave Radio Equipment, STM-1, 11,8 GHZ (1+1), (3+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 3 3 3 1 |  |
| 07-03-06104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 5 |  |
| 07-03-06105 07-03-06106 07-03-06107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 5 1 1 |  |
| 07-03-06108 07-03-06109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(4+1)configuration, 48V, 300Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-06110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-06111 07-03-06112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-06113 07-03-06114 07-03-06115 07-03-06116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-06117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-06118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-06119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-06120 07-03-06121 07-03-06222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital Microwave Link  Between  BAGHDAD &BASRAH  With

Extension To Port  UM QASR

Site - Name : DIWANIYA

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-07100 07-03-07101 07-03-07102 07-03-07103 | SDH Microwave Radio Equipment, STM-1, 8GHZ (3+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-07104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 4 |  |
| 07-03-07105 07-03-07106 07-03-07107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 4 1 1 |  |
| 07-03-07108 07-03-07109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(3+1)configuration, 48V, 300Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-07110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-07111 07-03-07112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-07113 07-03-07114 07-03-07115 07-03-07116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-07117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-01-07118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-07119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-07120 07-03-07121 07-03-07222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital Microwave Link Between BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name : ABU TABIK  REP.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-08100 07-03-08101 07-03-08102 07-03-08103 | SDH Microwave Radio Equipment, STM-1, 8 GHZ (3+1) Frequency & Space Diversity Radio Repeter All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-08104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 4 |  |
| 07-03-08105 07-03-08106 07-03-08107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 4 1 1 |  |
| 07-03-08108 07-03-08109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 200Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-08110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-08111 07-03-08112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-08113 07-03-08114 07-03-08115 07-03-08116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot Lot Lot | 0 0 0 0 |  |
| 07-03-08117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-08118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 0 |  |
| 07-03-08119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-08120 07-03-08121 07-03-08222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 0 0 0 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name  : SAMAWA T.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-09100 07-03-09101 07-03-09102 07-03-09103 | SDH Microwave Radio Equipment, STM-1,11,8GHZ (1+1), (3+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 3 3 3 1 |  |
| 07-03-09104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 5 |  |
| 07-03-09105 07-03-09106 07-03-09107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 5 1 1 |  |
| 07-03-09108 07-03-09109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(4+1)configuration, 48V, 300Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-09110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-09111 07-03-09112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-09113 07-03-09114 07-03-09115 07-03-09116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-09117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-09118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-09119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-09120 07-03-09121 07-03-09222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name  : DARADJHI   REP.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-10100 07-03-10101 07-03-10102 07-03-10103 | SDH Microwave Radio Equipment, STM-1, 8 GHZ (3+1) Frequency & Space Diversity Radio Repeter All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed inRack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-10104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 4 |  |
| 07-03-10105 07-03-10106 07-03-10107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 4 1 1 |  |
| 07-03-10108 07-03-10109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 200Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-10110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-10111 07-03-10112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-10113 07-03-10114 07-03-10115 07-03-10116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 0 0 |  |
| 07-03-10117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-10118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-10119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-10120 07-03-10121 07-03-10222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 0 0 0 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital Microwave Link Between  BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name  : NASSYRIAH

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-11100 07-03-11101 07-03-11102 07-03-11103 | -SDH Microwave Radio Equipment, STM-1, 11,8GHZ (1+1), (3+1) Frequency Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 3 3 3 1 |  |
| 07-03-11104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 5 |  |
| 07-03-11105 07-03-11106 07-03-11107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 5 1 1 |  |
| 07-03-11108 07-03-11109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(4+1)configuration, 48V, 300Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-11110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-11111 07-03-11112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-11113 07-03-11114 07-03-11115 07-03-11116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-11117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-01-11118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-11119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-11120 07-03-11121 07-03-11222 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital Microwave Link  Between  BAGHDAD & BASRAH With

Extension  To Port  UMQASR

Site - Name  : SUOK AL CHIOUH

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-12100 07-03-12101 07-03-12102 07-03-12103 | SDH Microwave Radio Equipment, STM-1,11 ,8 , 6 GHZ (1+1),(3+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals ( Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 3 3 3 1 |  |
| 07-03-12104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 4 |  |
| 07-03-12105 07-03-12106 07-03-12107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 4 1 1 |  |
| 07-03-12108 07-03-12109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(4+1)configuration, 48V, 300Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-12110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-12111 07-03-12112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-12113 07-03-12114 07-03-12115 07-03-12116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subracks with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoder-Decoder Equipment 34Mb/s ip/op., Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-12117 | -Network Management Equipment consisting of Local Network Management System platform (Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-12118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-12119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-12120 07-03-12121 07-03-12122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital Microwave Link Between BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name  : GABAYCHIA   REP.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-13100 07-03-13101 07-03-13102 07-03-13103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (3+1) Frequency & Space Diversity Radio Repeater All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-13104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 4 |  |
| 07-03-13105 07-03-13106 07-03-13107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 4 1 1 |  |
| 07-03-13108 07-03-13109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 200Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-13110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-13111 07-03-13112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-13113 07-03-13114 07-03-13115 07-03-13116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 0 0 |  |
| 07-03-13117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-01-13118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 0 |  |
| 07-03-13119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-13120 07-03-13121 07-03-13122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 0 0 0 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave  Link Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM QASR

Site - Name  : NIKAIYLA  REP.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-14100 07-03-14101 07-03-14102 07-03-14103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (3+1) Frequency & Space Diversity Radio Repeater All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-14104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 4 |  |
| 07-03-14105 07-03-14106 07-03-14107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 4 1 1 |  |
| 07-03-14108 07-03-14109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(3+1)configuration, 48V, 200Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-14110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-14111 07-03-14112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-14113 07-03-14114 07-03-14115 07-03-14116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 0 0 |  |
| 07-03-14117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-14118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 0 |  |
| 07-03-14119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-14120 07-03-14121 07-03-14122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 0 0 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave  Link  Between  BAGHDAD & BASRAH With

Extension  To  Port  UM QASR

Site - Name  : BASRAH  T.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-15100 07-03-15101 07-03-15102 07-03-15103 | SDH Microwave Radio Equipment, STM-1, 6,11GHZ (3+1) Frequency & SpaceDiversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 3 3 3 1 |  |
| 07-03-15104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 3 |  |
| 07-03-15105 07-03-15106 07-03-15107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 3 1 1 |  |
| 07-03-15108 07-03-15109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(4+1)configuration, 48V, 300Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-15110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-15111 07-03-15112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-15113 07-03-15114 07-03-15115 07-03-15116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoder-Decoder with 34Mb/s ip/op Access &Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire,Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-15117 | -Network Management Equipment consisting of Local Network Management System platform , (Hard ware), System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-15118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-15119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-15120 07-03-15121 07-03-15122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH With

Extension  To  Port  UM QASR

Site - Name  : BASRAH  EXGH.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-16100 07-03-16101 07-03-16102 07-03-16103 | SDH Microwave Radio Equipment, STM-1, 11,6GHZ (1+1), (3+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-16104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 2 |  |
| 07-03-16105 07-03-16106 07-03-16107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 2 1 1 |  |
| 07-03-16108 07-03-16109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(3+1)configuration, 48V, 200Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-16110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-16111 07-03-16112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-16113 07-03-16114 07-03-16115 07-03-16116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoder-Decoder 34Mb/s ip/op. Access & Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-16117 | -Network Management Equipment consisting of Local Network Management System platform (Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-16118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-16119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-16120 | Self Support Steel 80M. Tower For 4 Antennas , Complete With Grounding, Lightening Rod, Obstruction In Lighting, Painting & Installation Materials. | Lot | 1 |  |
| 07-03-16121 07-03-16122 07-03-16123 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 2 4 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave  Link  Between BAGHDAD & BASRAH With

Extension  To  Port  UM QASR

Site - Name  : KERBALAH

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-17100 07-03-17101 07-03-17102 07-03-17103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (1+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-17104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 2 |  |
| 07-03-17105 07-03-17106 07-03-17107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 2 1 1 |  |
| 07-03-17108 07-03-17109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 100Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-17110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-17111 07-03-17112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-17113 07-03-17114 07-03-17115 07-03-17116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-17117 | -Network Management Equipment consisting of Local Network Management System platform (Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-17118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-17119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-17120 07-03-17121 07-03-17122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH With

Extension  To  Port  UM QASR

Site - Name  : NADJAF

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-18100 07-03-18101 07-03-18102 07-03-18103 | SDH Microwave Radio Equipment, STM-1, 11GHZ (1+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-18104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-18105 07-03-18106 07-03-18107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-18108 07-03-18109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 100Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-18110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-18111 07-03-18112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-18113 07-03-18114 07-03-18115 07-03-18116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-18117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-18118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-18119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-18120 07-03-18121 07-03-18122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave  Link Between  BAGHDAD & BASRAH With

Extension  To Port  UM QASR

Site - Name  : SAMAWA  EXGH.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-19100 07-03-19101 07-03-19102 07-03-19103 | SDH Microwave Radio Equipment, STM-1, 11GHZ (1+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-19104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-19105 07-03-19106 07-03-19107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-19108 07-03-19109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 100Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-19110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-19111 07-03-19112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-19113 07-03-19114 07-03-19115 07-03-19116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-19117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-19118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-19119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-19120 07-03-19121 07-03-19122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link Between  BAGHDAD & BASRAH With

Extension  To  Port  UM QASR

Site - Name  : NASSYRIAH  EXGH.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-20100 07-03-20101 07-03-20102 07-03-20103 | SDH Microwave Radio Equipment, STM-1, 11GHZ (1+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-20104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-20105 07-03-20106 07-03-20107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-20108 07-03-20109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 100Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-20110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-20111 07-03-20112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-20113 07-03-20114 07-03-20115 07-03-20116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated),Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-20117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-20118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-20119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-20120 07-03-20121 07-03-20122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  : SALHIYA  T.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-21100 07-03-21101 07-03-21102 07-03-21103 | SDH Microwave Radio Equipment, STM-1, 11GHZ (3+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-21104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-21105 07-03-21106 07-03-21107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-21108 07-03-21109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(3+1)configuration, 48V, 200Ah Lead- Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-21110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-21111 07-03-21112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-21113 07-03-21114 07-03-21115 07-03-21116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoder-Decoder 34Mb/s ip/op Access & Connection Modules, Full Matrix(duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-21117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-21118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-21119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-21120 07-03-21121 07-03-21122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  : SOUK AL CHIOUH  T.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-22100 07-03-22101 07-03-22102 07-03-22103 | SDH Microwave Radio Equipment, STM-1, 11GHZ (1+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-22104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-22105 07-03-22106 07-03-22107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-22108 07-03-22109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 75Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-22110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-22111 07-03-22112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-22113 07-03-22114 07-03-22115 07-03-22116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoder-Decoder 34Mb/s ip/op Access & Connection Modules, Full Matrix(duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-22117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-22118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-22119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-22120 07-03-22121 07-03-22122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  :  BABYLON  T.

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-23100 07-03-23101 07-03-23102 07-03-23103 | SDH Microwave Radio Equipment, STM-1, 11GHZ (1+1) Frequency Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-23104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-23105 07-03-23106 07-03-23107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-23108 07-03-23109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 200 Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-23110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-23111 07-03-23112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-23113 07-03-23114 07-03-23115 07-03-23116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) V-S Encoder-Decoder Equipment 34Mb/s ip/op Access & Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 0 |  |
| 07-03-23117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-23118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 0 |  |
| 07-03-23119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-23120 07-03-23121 07-03-23122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  :  ZUBAIR

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-24100 07-03-24101 07-03-24102 07-03-24103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (1+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-24104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 2 |  |
| 07-03-24105 07-03-24106 07-03-24107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 2 1 1 |  |
| 07-03-24108 07-03-24109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 165Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-24110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-24111 07-03-24112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-24113 07-03-24114 07-03-24115 07-03-24116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-24117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-24118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-24119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-24120 | Self Support Steel 80M. Tower For 4 Antennas ( 3M. Dia.) Complete With Grounding, Lightening Rod, Obstruction Lighting , Painting & Installation Materials. | Lot | 1 |  |
| 07-03-24121 07-03-24122 07-03-24123 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  :  KHOUR  AL  ZUBAIR

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-25100 07-03-25101 07-03-25102 07-03-25103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (1+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-25104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 2 |  |
| 07-03-25105 07-03-25106 07-03-25107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 2 1 1 |  |
| 07-03-25108 07-03-25109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 130Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-25110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-25111 07-03-25112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-25113 07-03-25114 07-03-25115 07-03-25116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-25117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-25118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-25119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-25120 07-03-25121 07-03-25122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 1 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  :  UM QASR

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-26100 07-03-26101 07-03-26102 07-03-26103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (1+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 1 1 1 1 |  |
| 07-03-26104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 1 |  |
| 07-03-26105 07-03-26106 07-03-26107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-26108 07-03-26109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 100Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-26110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-26111 07-03-26112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 1 1 |  |
| 07-03-26113 07-03-26114 07-03-26115 07-03-26116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 1 1 |  |
| 07-03-26117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-26118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-26119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-26120 | Self Support Steel 80M. Tower For 4 Antennas ( 3M. Dia.) Complete With Grounding, Lightening Rod, Obstruction Lighting , Painting & Installation Materials. | Lot | 1 |  |
| 07-03-26121 07-03-26122 07-03-26123 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 1 2 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  : BAGHDAD  TRAINING  CENTER

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-27100 07-03-27101 07-03-27102 07-03-27103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (2+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-27104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 0 |  |
| 07-03-27105 07-03-27106 07-03-27107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-27108 07-03-27109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 200Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-27110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-27111 07-03-27112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 0 1 |  |
| 07-03-27113 07-03-27114 07-03-27115 07-03-27116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 0 0 |  |
| 07-03-27117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-27118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-27119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-27120 07-03-27121 07-03-27122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 2 4 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  : BAGHDAD  MAINTENANCE  CENTER

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-28100 07-03-28101 07-03-28102 07-03-28103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (2+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-28104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 0 |  |
| 07-03-28105 07-03-28106 07-03-28107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-28108 07-03-28109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 200Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-28110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-28111 07-03-28112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 0 1 |  |
| 07-03-28113 07-03-28114 07-03-28115 07-03-28116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 0 0 |  |
| 07-03-28117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-28118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-28119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-28120 07-03-28121 07-03-28122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 8 8 2 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX  SEVEN  / Telecommunications

Sub - sector : Digital  Microwave Link  Between  BAGHDAD & BASRAH  With

Extension  To  Port  UM  QASR

Site - Name  : BASRAH  MAINTENANCE  CENTER

| Code | Item Description | Unit | Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-29100 07-03-29101 07-03-29102 07-03-29103 | SDH Microwave Radio Equipment, STM-1, 6 GHZ (2+1) Frequency & Space Diversity Radio Terminal All Complete with Transmitters, Receivers, 128 QAM Modems Modules ,Protection Switch, Engineering order wire channel, 2Mb/s auxiliary traffic, Interface to central Network management, Peripherals (Lap Top PC , ETC......), Installed in Rack with 48V. d/c unit (duplicated). -Installation materials. -Spare parts & spare units. -Documentation. | Lot Lot Lot Set | 2 2 2 1 |  |
| 07-03-29104 | Microwave Parabolic Dish (1.8m., 2.4m. )Antenna with (one/ two)Polarization's all complete with Connector Flanges, Tower Attachment, Installation materials & Documentation. | Lot | 0 |  |
| 07-03-29105 07-03-29106 07-03-29107 | -Microwave Elliptical waveguide Feeder Run, 11/13 GHZ band all complete with Pressure Window, Fixture clamps, Earthing Kits, Installation materials & Documentation. -Medium Capacity Dehydrator. -10% Spares. | Lot Unit Lot | 1 1 1 |  |
| 07-03-29108 07-03-29109 | -DC Power Plant Including Modular Switched Rectifier system 48V/30A. module(2+1)configuration, 48V, 200Ah Lead-Acid Sealed- Batteries (2sets), Cabinets, Installation materials, cables, tools & Documentation. -Spare Parts & Spare Units. | Lot Lot | 1 1 |  |
| 07-03-29110 | -Air Conditioners, Duplicated 18000 / 24000 BTU Unit All Complete With, Piping & installation materials & fixtures, Spare parts & Documentation. | Lot | 2 |  |
| 07-03-29111 07-03-29112 | AC Power Plant Including Diesel Engine Generator Capacity 50 KVA, Dual Set, all complete with, Daily Fuel Tank, Fan, Piping, Control cubicle & Alarms, Installation Materials, Documentation & Tools -Spare parts & PCB's. | Lot Lot | 0 1 |  |
| 07-03-29113 07-03-29114 07-03-29115 07-03-29116 | -SDH Multiplex Equipment Including STM-16 Mutliplexer , STM-4,STM-1Multiplexer Subrack with redundant common units ADM, Terminal configuration , Tributary & Elect. Aggregate Cards (3x34Mb/s, 21x2Mb/s,140/155Mb/s switch, 7x2Mb/s) Access &Connection Modules, Full Matrix (duplicated), Auxiliary /Engineering order wire, Installation Materials & Tools, Racks with power supply unit (duplicated ) and cable head, Digital Distribution Frames & Documentation for each . -Primary Multiplex ( PCM-30CH.) Subrack with common units & channel PCB's of different configs. Installed in Racks with power supply units, cable head & Digital Distribution Frames. -Spare parts & spare units. -Installation materials & tools. | Lot Lot | 0 0 |  |
| 07-03-29117 | -Network Management Equipment consisting of Local Network Management System platform ( Hard ware),System software packages all complete with craft terminals, Mediation , Interface, and connection cards & modules, Installation Materials & Tools, Documentation & Spare Parts for each item. | Lot | 1 |  |
| 07-03-29118 | Test Instruments including : Digital Multimeter, Frequency Counter, Power Meter , PCM Multiplex Tester , RF Sweep Generator ,VSWR Test Set, Scalar Network Analyzer , RF Spectrum Analyzer, SDH Analyzer for STM-1,TX / RX MW Link Analyzer, PDH Analyzer (2Mb/s),Oscilloscope 500 MHZ Dual channel, Vedio Test Set, RF Attenuators Fixed & Variable ( 0 - 60 )DB, Cables & Adapters Waveguide / Coax. Adapters & relevant Documentation. | Lot | 1 |  |
| 07-03-29119 | -Specific Tools including: Specific Maintenance Tools Station Tools, Special Tools for waveguide & General Installation Tools. | Lot | 1 |  |
| 07-03-29120 07-03-29121 07-03-29122 | -Factory Training Abroad. -Training In Iraq. -Supervision Services (Supplier Expert). | Trainee Trainee Expert | 2 4 1 |  |

Expected Delivery Time: 9 Months			Expected Implementation Time: 9 Months

ANNEX SEVEN/ Telecommunications

Sub-sector:   REPLACEMENT OF OLD CROSSBAR EXCHANGES

Site Name:    KERBALA EXCHANGE in KERBALA

| # Code | Item Description | # Unit | # Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-40000 07-03-40001 07-03-40002 07-03-40003 07-03-40004 07-03-40005 07-03-40006 07-03-40007 07-03-40008 07-03-40009 | SWITCHING EQUIPMENT for 15000 subscriber lines and 200 E1 digital interface: - Subscriber Connections - Trunk equipment - Tones and Signaling - Control Processors - Operation and Maintenance Processors - Peripherals - Racks and Ironworks - Installation and Maintenance tools ,Materials and Facilities - Documentation | Lot Lot Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 1 1 2 |  |
| 07-03-40010 07-03-40011 07-03-40012 07-03-40013 07-03-40014 07-03-40015 07-03-40016 07-03-40017 07-03-40018 07-03-40019 | POWER PLANT - Operation and Maintenance Processors - Peripherals - 48V power supplies - DC\DC Converters - Inverters 2KVA - 48V Batteries, 6400 AH - Cables - Installation and Maintenance tools and Materials - Documentation | Lot Lot Lot Lot Lot Set Set Set Lot Set | 1 1 1 1 1 1 2 1 1 2 |  |
| 07-03-40020 07-03-40021 07-03-40022 07-03-40023 | AIR CONDITIONERS - 60 kW 50Hz, 220\380V - Installation and Maintenance tools and Materials - Documentation | Lot Set Lot Set | 1 3 1 2 |  |
| 07-03-40024 07-03-40025 07-03-40026 07-03-40027 07-03-40028 07-03-40029 07-03-40030 07-03-40031 | Main Distribution Frames - Terminal Blocks - Over voltage protection - Over current protection - Ironwork - Installation tools - Cables & Jumper wire - Documentation | Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 2 |  |
| 07-03-40032 07-03-40033 07-03-40034 07-03-40035 07-03-40036 07-03-40037 | FIRE PROTECTION EQUIPMENT - Detection units - Control &Alarm Unit - Manual Distinguishers - Installation and Maintenance tools and Materials - Documentation | Lot Lot Lot Lot Lot Set | 1 1 1 1 1 2 |  |
| 07-03-40038 | False Floor – False Ceiling for switching room | Lot | 1 |  |
| 07-03-40039 | SPARE PARTS | Lot | 1 |  |
| 07-03-40040 07-03-40041 07-03-40042 07-03-40043 07-03-40044 | DIESEL ENGINE GENERATOR - Capacity 600 KVA - Installation and Maintenance tools and Materials - Spare Parts - Documentation | Lot Set Lot Lot Set | 1 2 1 1 2 |  |
| 07-03-40045 07-03-40046 07-03-40047 07-03-40048 | SERVICES - Training abroad - Training in Iraq - Supplier Experts(Supervisors or Trainer) | Lot Trainee Trainee Expert | 1 2 6 2 |  |

Expected Delivery Time: 5 Months			Expected Implementation Time: 6 Months

ANNEX SEVEN/ Telecommunications

Sub-sector: REPLACEMENT OF OLD CROSSBAR EXCHANGES

Site Name:    DIWANIYA  EXCHANGE in DIWANIYA

| # Code | Item Description | # Unit | # Qty. | Complementary Items |
| --- | --- | --- | --- | --- |
| 07-03-40049 07-03-40050 07-03-40051 07-03-40052 07-03-40053 07-03-40054 07-03-40055 07-03-40056 07-03-40057 07-03-40058 | SWITCHING EQUIPMENT for 15000 subscriber lines and 200 E1 digital interface: - Subscriber Connections - Trunk equipment - Tones and Signaling - Control Processors - Operation and Maintenance Processors - Peripherals - Racks and Ironworks - Installation and Maintenance tools ,Materials and Facilities - Documentation | Lot Lot Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 1 1 2 |  |
| 07-03-40059 07-03-40060 07-03-40061 07-03-40062 07-03-40063 07-03-40064 07-03-40065 07-03-40066 07-03-40067 07-03-40068 | POWER PLANT - Operation and Maintenance Processors - Peripherals - 48V power supplies - DC\DC Converters - Inverters 2KVA - 48V Batteries, 6400 AH - Cables - Installation and Maintenance tools ,Materials and Facilities - Documentation | Lot Lot Lot Lot Lot Set Set Set Lot Set | 1 1 1 1 1 1 2 1 1 2 |  |
| 07-03-40069 07-03-40070 07-03-40071 07-03-40072 | AIR CONDITIONERS - 60 kW 50Hz, 220\380V - Installation and Maintenance tools and Materials - Documentation | Lot Set Lot Set | 1 3 1 2 |  |
| 07-03-40073 07-03-40074 07-03-40075 07-03-40076 07-03-40077 07-03-40078 07-03-40079 07-03-40080 | Main Distribution Frames - Terminal Blocks - Over voltage protection - Over current protection - Ironwork - Installation tools - Cables & Jumper wire - Documentation | Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 2 |  |
| 07-03-40081 07-03-40082 07-03-40083 07-03-40084 07-03-40085 07-03-40086 | FIRE PROTECTION EQUIPMENT - Detection units - Control &Alarm Unit - Manual Distinguishers - Installation and Maintenance tools and Materials - Documentation | Lot Lot Lot Lot Lot Set | 1 1 1 1 1 2 |  |
| 07-03-40087 | False Floor – False Ceiling for switching room | Lot | 1 |  |
| 07-03-40088 | SPARE PARTS | Lot | 1 |  |
| 07-03-40089 07-03-40090 07-03-40091 07-03-40092 07-03-40093 | DIESEL ENGINE GENERATOR - Capacity 600 KVA - Installation and Maintenance tools and Materials - Spare Parts - Documentation | Lot Set Lot Lot Set | 1 2 1 1 2 |  |
| 07-03-40094 07-03-40095 07-03-40096 07-03-40097 | SERVICES - Training abroad - Training in Iraq - Supplier Experts(Supervisors or Trainer) | Lot Trainee Trainee Expert | 1 2 6 2 |  |

Expected Delivery Time: 5 Months			Expected Implementation Time: 6 Months

ANNEX SEVEN/ Telecommunications

Sub-sector: REPLACEMENT OF OLD CROSSBAR EXCHANGES

Site Name:    NASIRIYA  EXCHANGE  in NASIRIYA

| # Code | Item Description | # Unit | # Qty. | Comp. Items |
| --- | --- | --- | --- | --- |
| 07-03-40098 07-03-40099 07-03-40100 07-03-40101 07-03-40102 07-03-40103 07-03-40104 07-03-40105 07-03-40106 07-03-40107 | SWITCHING EQUIPMENT for 15000 subscriber lines and 200 E1 digital interface: - Subscriber Connections - Trunk equipment - Tones and Signaling - Control Processors - Operation and Maintenance Processors - Peripherals - Racks and Ironworks - Installation and Maintenance tools ,Materials and Facilities - Documentation | Lot Lot Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 1 1 2 |  |
| 07-03-40108 07-03-40109 07-03-40110 07-03-40111 07-03-40112 07-03-40113 07-03-40114 07-03-40115 07-03-40116 07-03-40117 | POWER PLANT - Operation and Maintenance Processors - Peripherals - 48V power supplies - DC\DC Converters - Inverters 2KVA - 48V Batteries, 6400 AH - Cables - Installation and Maintenance tools and Materials - Documentation | Lot Lot Lot Lot Lot Set Set Set Lot Set | 1 1 1 1 1 1 2 1 1 2 |  |
| 07-03-40118 07-03-40119 07-03-40120 07-03-40121 | AIR CONDITIONERS - 60 kW 50Hz, 220\380V - Installation and Maintenance tools and Materials - Documentation | Lot Set Lot Set | 1 3 1 2 |  |
| 07-03-40122 07-03-40123 07-03-40124 07-03-40125 07-03-40126 07-03-40127 07-03-40128 07-03-40129 | Main Distribution Frames - Terminal Blocks - Over voltage protection - Over current protection - Ironwork - Installation tools - Cables & Jumper wire - Documentation | Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 2 |  |
| 07-03-40130 07-03-40131 07-03-40132 07-03-40133 07-03-40134 07-03-40135 | FIRE PROTECTION EQUIPMENT - Detection units - Control &Alarm Unit - Manual Distinguishers - Installation and Maintenance tools and Materials - Documentation | Lot Lot Lot Lot Lot Set | 1 1 1 1 1 2 |  |
| 07-03-40136 | False Floor – False Ceiling for switching room | Lot | 1 |  |
| 07-03-40137 | SPARE PARTS | Lot | 1 |  |
| 07-03-40138 07-03-40139 07-03-40140 07-03-40141 07-03-40142 | DIESEL ENGINE GENERATOR - Capacity 600 KVA - Installation and Maintenance tools and Materials - Spare Parts - Documentation | Lot Set Lot Lot Set | 1 2 1 1 2 |  |
| 07-03-40143 07-03-40144 07-03-40145 07-03-40146 | SERVICES - Training abroad - Training in Iraq - Supplier Experts(Supervisors or Trainer) | Lot Trainee Trainee Expert | 1 2 6 2 |  |

Expected Delivery Time: 5 Months			Expected Implementation Time: 6 Months

ANNEX SEVEN/ Telecommunications

Sub-sector: REPLACEMENT OF OLD CROSSBAR EXCHANGES

Site Name:    ANDALUS EXCHANGE  in  BASRAH

| # Code | Item Description | # Unit | # Qty. | Comp. Items |
| --- | --- | --- | --- | --- |
| 07-03-40147 07-03-40148 07-03-40149 07-03-40150 07-03-40151 07-03-40152 07-03-40153 07-03-40154 07-03-40155 07-03-40156 | SWITCHING EQUIPMENT for 15000 subscriber lines and 200 E1 digital interface: - Subscriber Connections - Trunk equipment - Tones and Signaling - Control Processors - Operation and Maintenance Processors - Peripherals - Racks and Ironworks - Installation and Maintenance tools ,Materials and Facilities - Documentation | Lot Lot Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 1 1 2 |  |
| 07-03-40157 07-03-40158 07-03-40159 07-03-40160 07-03-40161 07-03-40162 07-03-40163 07-03-40164 07-03-40165 07-03-40166 | POWER PLANT - Operation and Maintenance Processors - Peripherals - 48V power supplies - DC\DC Converters - Inverters 2KVA - 48V Batteries, 6400 AH - Cables - Installation and Maintenance tools and Materials - Documentation | Lot Lot Lot Lot Lot Set Set Set Lot Set | 1 1 1 1 1 1 2 1 1 2 |  |
| 07-03-40167 07-03-40168 07-03-40169 07-03-40170 | AIR CONDITIONERS - 60 kW 50Hz, 220\380V - Installation and Maintenance tools and Materials - Documentation | Lot Set Lot Set | 1 3 1 2 |  |
| 07-03-40171 07-03-40172 07-03-40173 07-03-40174 07-03-40175 07-03-40176 07-03-40177 07-03-40178 | Main Distribution Frames - Terminal Blocks - Over voltage protection - Over current protection - Ironwork - Installation tools - Cables & Jumper wire - Documentation | Lot Lot Lot Lot Lot Lot Lot Set | 1 1 1 1 1 1 1 2 |  |
| 07-03-40179 07-03-40180 07-03-40181 07-03-40182 07-03-40183 07-03-40184 | FIRE PROTECTION EQUIPMENT - Detection units - Control &Alarm Unit - Manual Distinguishers - Installation and Maintenance tools and Materials - Documentation | Lot Lot Lot Lot Lot Set | 1 1 1 1 1 2 |  |
| 07-03-40085 | False Floor – False Ceiling for switching room | Lot | 1 |  |
| 07-03-40186 | SPARE PARTS | Lot | 1 |  |
| 07-03-40187 07-03-40188 07-03-40189 07-03-40190 07-03-40191 | DIESEL ENGINE GENERATOR - Capacity 600 KVA - Installation and Maintenance tools and Materials - Spare Parts - Documentation | Lot Set Lot Lot Set | 1 2 1 1 2 |  |
| 07-03-40192 07-03-40193 07-03-40194 07-03-40195 | SERVICES - Training abroad - Training in Iraq - Supplier Experts(Supervisors or Trainer) | Lot Trainee Trainee Expert | 1 2 6 2 |  |

Expected Delivery Time: 5 Months			Expected Implementation Time: 6 Months