| SCR 986 - Phase VI - OSRO/IRQ/903/DHA No.2 AGRO-INDUSTRY |  |  |
| --- | --- | --- |
| Rehabilitation of Erbil Baby-Food Processing Plant - Erbil Governorate |  |  |  |

| Code | Item Description | Quantity | Unit |
| --- | --- | --- | --- |
| ERBIL BABY-FOOD PROCESSING PLANT - Carrot puree preparation line (500 Kg/h) |  |  |  |
| 01-14-00001 | Elevator: Inclined belt conveyor manufactured in food grade synthetic rubber with stainless steel buckets and hopper. | 1 | Item |
| 01-14-00002 | Rotating washer: equipped with showers and rubbing facilities for tubers. | 1 | Item |
| 01-14-00003 | Rotating washer: equipped with showers. | 2 | Item |
| 01-14-00004 | Sorting device for carrot tipping. Equipped with belt conveyor made in appropriate food degree rubber and speed variator. | 1 | Item |
| 01-14-00005 | Abrasion peeling system for tubers and carrots, particularly. | 1 | Item |
| 01-14-00006 | Sorting line for tubers. Equipped with belt conveyor made in appropriate food grade rubber and speed variator. | 1 | Item |
| 01-14-00007 | Bucket elevator: inclined belt conveyor made in food grade synthetic rubber and with stainless steel buckets. | 1 | Item |
| 01-14-00008 | Chopper: suitable to achieve a thin chopping of carrots by means of the combined action of different tools. | 1 | Item |
| 01-14-00009 | Continuous cooker: thermobreack type, suitable for prechopped fruit baby-food products, particularly. Thermomechanic process using direct and/or indirect steam. | 1 | Item |
| 01-14-00010 | 120 L tank: made in stainless steel AISI 304. Sloped bottom type, fitted with level regulator for the automatic start up and stop of the service pump | 1 | Item |
| 01-14-00011 | Volumetric pump: manufactured in stainless steel. Positive pump, with stainless steel screw rotor and food grade synthetic rubber stator. Driven by mechanical speed variator. | 1 | Item |
| 01-14-00012 | Pulper: with beaters and sieve to process carrots. Equipped with hopper and screw, washing device, sieve pushing cover. | 2 | Item |
| 01-14-00013 | 200 L tank with wheels | 4 | Item |
| 01-14-00014 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. Complete with electromechanical programming device suitable for automatic production processes. | 1 | Item |
| 01-14-00015 | Fittings: Accessories, connections, taps, valves, piping, joints, etc. made in stainless steel; working platforms. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Additional machine for leafy vegetables (capacity up to 500 Kg/h) |  |  |  |
| 01-14-00016 | Sorting line: for washing vegetables (leaves), with belt conveyor made in PVC. Lateral plans for cut and selection, cleaning system, working table, stairs and footboards. | 1 | Item |
| 01-14-00017 | Drum separator: to discharge wastes and other materials. Equipped with rain washing system. | 1 | Item |
| 01-14-00018 | Washing device: with belt elevator, blower air distribution system, water recycling pump, brushes and showers.. | 1 | Item |
| 01-14-00019 | Washing device: like above and complete with steam injector, valves and manual steam regulator with thermometer. | 1 | Item |
| 01-14-00020 | Disk cutter with belt ocnveyor: to receive the leaves and cut them in order to reduce the volume. | 1 | Item |
| 01-14-00021 | Screw conveyor: to collect the cutted leaves | 1 | Item |
| 01-14-00022 | 200 L tank: with wheels | 4 | Item |
| 01-14-00023 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. | 1 | Item |
| 01-14-00024 | Fittings: Accessories, connections, taps, valves, piping, joints, etc. made in stainless steel; working platforms. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Meat cooking line (capacity 500 Kg/h) |  |  |  |
| 01-14-00025 | 200 L Batch pans on wheels, with earthed up scale | 5 | Item |
| 01-14-00026 | Graund scale: capacity up to 500 KG | 1 | Item |
| 01-14-00027 | Lifter: to lift/tilt the pans into the cooking kettles. | 2 | Item |
| 01-14-00028 | Up to 800 L cooking kettles: fixed type. Equipped with special stirrer for baby-food. Manufactured in stainlees steel AISI 304. Fittings and valves. | 2 | Item |
| 01-14-00029 | Water group: automatic dosing group up to 10000L/h | 2 | Item |
| 01-14-00030 | Scaffolding: structure to support processing equipment. Made in stainless steel, aluminium and carbon steel. | 1 | Set |
| 01-14-00031 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. Complete with electromechanical programming device suitable for automatic production processes. | 1 | Item |
| 01-14-00032 | Fittings: Accessories, connections, taps, valves, piping, joints, etc. made in stainless steel; working platforms. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Mixing line: puree, sugar syrup, micro-nutrients (500 L/h) |  |  |  |  |
| 01-14-00033 | Pneumatic pumps: to transfer thick and viscous liquids from aseptic aluminium bags to the stocking tank. Made in stainless steel AISI 304. | 2 | Item |
| 01-14-00034 | 1000 L mixing tank: made in stainless steel AISI 304. Equipped with: slow eccentric stirrer, level indicator with transparent pipe, all required fittings. | 2 | Item |
| 01-14-00035 | Volumetric pump: manufactured in stainless steel. Positive pump, with stainless steel screw rotor and food grade synthetic rubber stator. Driven by mechanical speed variator. | 2 | Item |
| 01-14-00036 | Water group: automatic dosing group up to 10000L/h | 1 | Item |
| 01-14-00037 | 1000 L mixing tank: made in stainless steel AISI 304. Equipped with: slow eccentric stirrer, level indicator with transparent pipe, all required fittings. | 1 | Item |
| 01-14-00038 | Scaffolding: structure to support processing equipment. Made in stainless steel, aluminium and carbon steel. | 1 | Set |
| 01-14-00039 | Volumetric pump: manufactured in stainless steel. Positive pump, with stainless steel screw rotor and food grade synthetic rubber stator. Driven by mechanical speed variator. Filtering group is included. | 1 | Item |
| 01-14-00040 | Dosing preparation group: for baby-food product. Consisting of steril mixing tank fitted with magnetic sensors for presetting the weighing cycle. | 1 | Item |
| 01-14-00041 | Electro-pneumatic valve group: equipped with filter and by-pass to intercept the water flux in order to feed, in appropriate way, the dosing preparation group. | 1 | Item |
| 01-14-00042 | Heating/cooling tank: made in stainless steel AISI 304. Thermoinsulated tank equipped with plate excenger to heat or cool the product by means of steam or water circulation; cover, eccentric stirrer. | 2 | Item |
| 01-14-00043 | Centrifugal pumps: made in stainlees steel 316, coupled with motor. | 2 | Item |
| 01-14-00044 | 200 L plastic tank: for mixing purpose. | 1 | Item |
| 01-14-00045 | Centrifugal pump: made in stainlees steel 316, coupled with motor. Equipped with filtering group. | 1 | Item |
| 01-14-00046 | Hot water preparation system | 1 | Item |
| 01-14-00047 | Heated table: made in stainless steel AISI 304. Equipped with plates for hetaing/cooling product like butter | 1 | Item |
| 01-14-00048 | Butter fuser: of adequate capacity | 1 | Item |
| 01-14-00049 | 200 L tank: with wheels. | 2 | Item |
| 01-14-00050 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. Complete with electromechanical programming device suitable for automatic production processes. | 1 | Item |
| 01-14-00051 | Fittings: Accessories, connections, taps, valves, piping, joints, etc. made in stainless steel; working platforms. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Mixing Line |  |  |  |
| 01-14-00052 | 600 L heated mixing tank: made in stainless steel AISI 304. Equipped with: slow eccentric stirrer, level indicator with transparent pipe, all required fittings. | 2 | Item |
| 01-14-00053 | Volumetric pump: manufactured in stainless steel. Positive pump, with stainless steel screw rotor and food grade synthetic rubber stator. Driven by mechanical speed variator. | 2 | Item |
| 01-14-00054 | Colloidal mill:for fine grinding and homogenizing baby-food product, particularly. | 2 | Item |
| 01-14-00055 | 600 L heated mixing tank: made in stainless steel AISI 304. Equipped with: special stirrer for baby-food, level indicator with transparent pipe, all required fittings. | 1 | Item |
| 01-14-00056 | Volumetric pump: manufactured in stainless steel. Positive pump, with stainless steel screw rotor and food grade synthetic rubber stator. Driven by mechanical speed variator. | 1 | Item |
| 01-14-00057 | Deaerator: to eliminate air incorporated into the product by nebulization under vacuum and rotating disk. The aromatic steams are condensed and re-incorporated into the product by a recovery device. | 1 | Item |
| 01-14-00058 | Pre-heater: Heat excenger with scraped vertical surface for heating/cooling product operations. Equipped with motor and speed variator. | 1 | Item |
| 01-14-00059 | Piping metal detector: to eliminate metallic particules from the product. | 1 | Item |
| 01-14-00060 | Double filtering group | 1 | Item |
| 01-14-00061 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. Complete with electromechanical programming device suitable for automatic production processes. | 2 | Item |
| 01-14-00062 | Fittings: Accessories, connections, taps, valves, piping, joints, etc. made in stainless steel; working platforms. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Filling Line (new and/or recycled-sterile glass jars: 5000 jars/h of 100 g.) |  |  |  |
| 01-14-00063 | Rotating table: for temporary accumulation of jars to feed filling line. | 1 | Item |
| 01-14-00064 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. | 1 | Item |
| 01-14-00065 | Washer/blower: for recycled/used glass jars. 4 different steps should be provided: pre-washing, washing (hot water with detergent), rinsing (cold water) and final rinsing. Steam instrumentation and fittings. Blower for new jars. | 1 | Item |
| 01-14-00066 | Ranging belt conveyor: suitable for jars. Adjustable sides and speed variator. | 1 | Item |
| 01-14-00067 | Inspection device: for the electronic checking of the jars before filling operations. The system should work by means of a camera. | 1 | Item |
| 01-14-00068 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. | 1 | Item |
| 01-14-00069 | Tunnel pre-heater: to heat empty jars in order to avoid thermal choc of glass jars during filling operations. Steam operated. | 1 | Item |
| 01-14-00070 | Volumetric filler: for baby-food. Two filling head for 100 g to 1000 g jars. | 1 | Item |
| 01-14-00071 | Automatic capper: to close jars in a steam vacuum environment. Capacity up to 6000 caps/h. Suitable to close caps type: deep twist-off, regular and medium twist-off, pry-off, press-on, combo plastic-twist and pry-twist. Electronic device to measure vacuum level. | 1 | Item |
| 01-14-00072 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. | 1 | Item |
| 01-14-00073 | Basket loader: semi-automatic loader (for square baskets). Consisting of: conveyor for basket feeding; row pusher; plastic guide to facilitate moving. | 1 | Item |
| 01-14-00074 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. | 1 | Item |
| 01-14-00075 | Fittings: Accessories, connections, taps, valves, piping, joints, reducers, bends, tees, manual sanitary valves, pipe holders and support etc. made in stainless steel to connect utilities to the process equipment. Working platforms are also included. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Sterilization line |  |  |  |
| 01-14-00076 | Sterilizing autoclave: horizontal type, completely automatic. Designed to cover plant requirements. Micro, mechanical and pressostatic safety devices are required. Equipped with all fittings and accessories, with instruments for temperature/pressure/time recording data including adequate amount of recording paper, and electric-pneumatic control board. Adequate amount of baskets and inserts for the baskets, made in stainless steel, are required. | 3 | Item |
| ERBIL BABY-FOOD PROCESSING PLANT - Quarantine line |  |  |  |
| 01-14-00077 | Basket unloader: semi-automatic unit for square baskets. Consisting of: conveyor for layer discharging; pusher; lifting group; motor with speed variator; electric board. | 1 | Item |
| 01-14-00078 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. | 1 | Item |
| 01-14-00079 | Washer/Drier unit: suitable to wash and dry filled jars before labelling. Equipped with thermostat to adjust themperature | 1 | Item |
| 01-14-00080 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. Special device to eliminate jars with insufficient vacuum level. | 1 | Item |
| 01-14-00081 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. | 1 | Item |
| 01-14-00082 | Palletizer: for filled jars. Jars will be palletized for inspection. Capacity more than 6000 jars/h. | 1 | Item |
| 01-14-00083 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. | 1 | Item |
| 01-14-00084 | Fittings: Accessories, connections, taps, valves, piping, joints, reducers, bends, tees, manual sanitary valves, pipe holders and support etc. made in stainless steel to connect utilities to the process equipment. Working platforms are also included. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Final packaging line |  |  |  |
| 01-14-00085 | Depalletizer: for inspected filled jars. Capacity more than 6000 jars/h. | 1 | Item |
| 01-14-00086 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. Special device to eliminate jars with insufficient vacuum level. | 1 | Item |
| 01-14-00087 | Belt conveyor: suitable for jars. Adjustable sides and speed variator. | 1 | Item |
| 01-14-00088 | Labelling machine: automatic machine equpped with fix store, cold glue. Wrapping body label on cylindrical glass containers of 100 g. | 1 | Item |
| 01-14-00089 | Ink jet coding machine: one head | 1 | Item |
| 01-14-00090 | Sleever: equipped with unroller of simple reel; retracting tunnel with automatic and pneumatic raising. Including consumable material for sleever for one year of operation. | 1 | Set |
| 01-14-00091 | Belt conveyor: for cartoning | 1 | Item |
| 01-14-00092 | Roller: Non motorized roller system | 1 | Item |
| 01-14-00093 | Rotating table: for temporary accumulation of jars. | 1 | Item |
| 01-14-00094 | Electric control board: made in stainless steel and including main automatic switch, on/off switches, warning lights, automatic overload cutout to protect the motors. | 1 | Item |
| 01-14-00095 | Fittings: Accessories, connections, taps, valves, piping, joints, reducers, bends, tees, manual sanitary valves, pipe holders and support etc. made in stainless steel to connect utilities to the process equipment. Working platforms are also included. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - C.I.P. Washing system |  |  |  |
| 01-14-00096 | CIP station: Cleaning-in-place station suitable to wash by means of hot basic detergent (75°C), hot acid detergent (60°C) and to rinse by means of water with economizer. Equipped with two thermo-insulated mixing tanks; water economizing tank; pressure pump; system to heat basic and acid detergent, adjust its concentration and recover the detergent; distribution pneumatic valves operated by programmer; electric control board with synoptic display. | 1 | Item |
| 01-14-00097 | Centrifugal pump: coupled with electric motor. Made in stainless steel AISI 316 | 5 | Item |
| 01-14-00098 | Hydrocleaner: movable group working with hot water (170 bar up to 85°C) or steam (50 bar up to 150°C). Equipped with pressure pump, burner, spraying gun and pipe. | 2 | Item |
| 01-14-00099 | Fittings: Accessories, connections, taps, valves, piping, joints, reducers, bends, tees, manual sanitary valves, pipe holders and support etc. made in stainless steel to connect utilities to the process equipment. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Miscellaneous equipment and material |  |  |  |
| 01-14-00100 | Twenty tons weighbridge: supplied with recorded weighting card for gross weight and empty truck. | 1 | Item |
| 01-14-00101 | Floor scale: to weigh raw material, ingredients and finished products. Capacity Kg 1000, division 500 g. | 3 | Item |
| 01-14-00102 | Scale to weigh ingredients: capacity 100, division 500 g | 1 | Item |
| 01-14-00103 | Trans-pallet: Hydraulic, manually operated, portable trolley to lift, move and shift up to 1.5 ton load on pallets. Manufactured with two fixed and one swivel wheel, hydraulic pump, oil. | 2 | Unit |
| 01-14-00104 | Fork lift: battery operated truck to move/shift/lift three tons of palletized load and to stack the pallets at 3.2 m (4 height). Short turn in radius type. Equipped with double set of battery and battery re-charger. | 1 | Item |
| 01-14-00105 | Mechanical tool box with: pliers, pincers, hammers, hacksaw, spanners (ring and open), adjustable wrenches, vice, screw drivers, torque wrench, hole punch, gasket cutter, flaring tools, grease guns. | 2 | Set |
| 01-14-00106 | Electrical tool box with: pliers, screw drivers, pulley, gloves, voltmeter, ammeter, tong tester, lamp tester, fuse coil, connectors, lugs terminals, hacksaw, portable drilling machine. | 2 | Set |
| 01-14-00107 | Plumber tool box with: wrenches, BSP dies, tools from 12 to 100 mm, gasket cutter, pipe cutter, hacksaw, pipe chain, vice, Teflon tape. | 2 | Set |
| 01-14-00108 | Office furniture and equipment: Desk, chairs, cabinet, calculator, photocopier, fax machine, stationary, etc.. | 1 | Set |
| 01-14-00109 | Administrative and marketing data processing equipment: Computer type Pentium II equipped with accumulator/stabilizer, printer and adequate programs. | 1 | Set |
| 01-14-00110 | Detergent: Adequate amount of detergent for, CIP station, sanitary movable machine, and in particular: acidic disinfectant based on hydrogen peroxide and parasitic acid (P3-oxonia active 30); acidic disinfectant based on inorganic acid-phosphoric- and inhibitors of corrosion (P3-PE4 special A); detergent based on caustic alkalis, special polyphosphates and organic sequestrates (P3-super CS); and as well specific detergent required for baby food equipment and materials. | 1 | Set |
| 01-14-00111 | Glue: Adequate amount of glue for carton (packing) and labels. | 400 | Kg. |
| 01-14-00112 | Seasoned wooden pallets: for mechanical handling of cartons, raw material crates and finished products. | 300 | Unit |
| 01-14-00113 | Packaging material: 100 g glass jars with lids (two colors). | 3,000 | 1,000 pack |
| 01-14-00114 | Packaging material: labels for several products | 3,000 | 1,000 pack |
| 01-14-00115 | Micro-nutrients: starch | 2,000 | Kg. |
| 01-14-00116 | Micro-nutrients: citric acid | 1,000 | Kg. |
| 01-14-00117 | Micro-nutrients: L-ascorbic acid | 1,000 | Kg. |
| 01-14-00118 | Micro-nutrients: Poly-vitaminic compound | 250 | Kg. |
| 01-14-00119 | Cartons: of adequate size to pack 80 to 100 jars (in two layers), with separator. | 10,000 | Unit |
| 01-14-00120 | Polyethylene wrapping material: for pallets | 2,000 | Rolls |
| ERBIL BABY-FOOD PROCESSING PLANT - Laboratory |  |  |  |
| 01-14-00121 | Equipment: No. 4 s.s. Refractometers from 0 to 35 and from 40 to 80 B; No.1 s.s. Refracolor refractometer 0 to 50; No. 1 Unirefrex refractometer; No. 1 Reflexometer; No.1 essential oil distiller; No.1 glass miniconcentrator complete Rotavapor; No.1 analytical electronic scale, 80 g capacity and with a sensitivity of 1/tenth of mmg, digital reading; No.1 table centrifuge up to 5300 rpm, and equipped with 24 test-tubes of 10 cc each; No.1 Bricktester for testing cardboard seal; No.2 hand vacuum-meters for testing cans; No.1 can tester for testing seaming; No.1 jar tester for testing caps sealing; No. 1 table pH-meter, with microprocessor for measuring pH, mV and temperature. Standard scale display; No.1 thermostatic stove; No.1 consistometer; No.1 magnetic mixer with heating plate, speed regulator and magnetic anchor; No.1 "Aercontrol" for oxygen residue determination, No. 1 SO2 analyzer, No.1 complete Kit for bacteriological analyses, No. 1 distilled water generator; No.1 glass and material sterilizer. | 1 | Set |
| 01-14-00122 | Glass: No. 1 glass kit complete of: No. 10 glasses, No. 5 flasks, No. 4 graduated cylinders, Burettes all in Pyrex, No. 1 Bunsen, No. 1 set of Pasteur pipettes, No. 1 set of graduated pipettes, No. 2 funnels, No. 2 parafilm, No. 2 thermometers, No. 2 set of required glasses for bacteriological analysis. | 1 | Set |
| 01-14-00123 | Chemicals: all required chemicals to conduct the analysis for two years. | 1 | Set |
| ERBIL BABY-FOOD PROCESSING PLANT - Utilities system |  |  |  |
| 01-14-00124 | Electric power generation: complete diesel air cooled generator unit with automatic change over system in case of main power failure. Pumps, piping, valves, joints, breaker, reservoir, fittings are included.Capacity of 1.5 time plant requirements, but not less than 500 KVA. | 1 | Set |
| 01-14-00125 | Electrical power distribution: including transformer to step down voltage to 410-420; main distribution board with incoming and outgoing Switch Fuse Unit to individual plant sub-boards and to lighting etc, as well as all other non-plant power consumption points; electric wiring to all process equipment and non-plant power consumption points. Capacitors for power factor improvement. | 1 | Set |
| 01-14-00126 | Compressed air system: including 2 Nos. 75% capacity single/double stage reciprocating air compressors, oil-free/moisture-free; No. 1 high pressure receiver of adequate pressure and volume capacity; distribution material with piping, valves, drum oil traps, gauges, etc.. | 1 | Set |
| 01-14-00127 | Steam generation and distribution: complete furnace steam generation/distribution and control system, using regular highway Diesel as fuel. Capacity of 4,000 Tons/h,10 bar. Fire tube steam boiler with horizontal body, valves, regulation and control panel, safety devices, electric control panel, water feeding group with tank of adequate capacity. The supply will include: heavy oil burner, water softening treatment, single wall chimney in stainless steel, steam manifold and distribution network, condensate recovery system. | 1 | Set |
| 01-14-00128 | Cooling tower system : including blowers , supports, pumps,piping, joints and all required fittings adquate to recycle (150 m3/h). | 1 | Set |
| 01-14-00129 | Potable water preparation group: for water processing utilization. A treatment cycle should include at least: a first rough filtration, a second one with activated carbon and a final clarification. Capacity 500 L/h. | 1 | Set |
| 01-14-00130 | Water /steam mixing group: equipped with nozzle and complete of all accessories. Working temperature of the water 20 to 70 °C. | 2 | Set |
| BABY FOOD PROCESSING PLANT - Spare parts |  |  |  |
| 01-14-00131 | Spare parts: According to the experience of the equipment manufacturer, required spare parts will be provided. Reference is made to processing equipment, laboratory, utilities system and miscellaneous equipment. | 1 | Lot |